MGL1211

Pre-stack depth migrated files mcs01, mcs02, mcs02a

Processing history

1. Geometry definition to bin the data into 6.25-m-spaced common mid-point gathers

2. Band pass filtering (3-7-220-250 Hz);

3. Trace editing;

4. Spherical divergence correction;

5. Resample to 4 ms;

6. Despike using LIFT method;

7. Radon filtering;

8. Predictive deconvolution;

9. Kirchhoff pre-stack depth migration and velocity analysis;

10. Automatic gain control for display purpose.